Embedded Systems: What are Embedded Systems?:  Computer-based systems that do not appear to be computers – complexity is hidden from the user. • They interact with users via simple interface They interact with another device(flash drive). Generic Embedded System Structure: (below). Efficient Rules: Price, Performance, Power, Maintainability, Functionality, Size, Time-to-market, Safety .Operating systems: Manages other programs, allows many programs to be executed together, incorporates a nice user interface, needs processing power and memory, slows down the system, makes development easier. User <-> Application <-> Operating System <-> Hardware. Embedded Linux systems: Embedded systems tend to be application-specific • Perform one task or set of related tasks. Higher design efficiency is possible. Embedded Operating Systems: It has basic components of an operating system: Process management • Memory management • File management • I/O management • Network management. Main characteristics • Configurable • Real-time.

Embedded Linux: Basic Linux commands: File Handling “mkdir” – make directories, “ls”:  list directory contents (“ls - l”  show more details such as permissions, ownership, time stamp was file being made). “cd” – changes directories. “pwd” – print name of current working directory.  “vim” – vi Improved, a programmers text editor • “cp” – copy files and directories • Usage: cp [OPTION] SOURCE DEST. “mv” – move (rename) files. • “rm” – remove files or directories.  Basic Linux commands: Text Processing:  “cat” – concatenate files and print on the standard output  (• cat –n file1.txt shows number line. “echo” – display a line of text • “grep” – print lines matching a pattern. Ex.  ifconfig | grep ‘inet addr: ’ “wc” – print the number of newlines, words, and bytes in files sort – sort lines of text files (by Ascii, Ascending order)  (sort –r” descending order) . Permission Level: “r” means “read only” permission • “w” means “write” permission • “x” means “execute” permission. rwx-rw-r– means owner has all three permissions, group has read and write, others have only read permission Read permission -4, write-2, execute -1 • ex, rwx-rw-r- = 764. Basic Linux commands: System Administration:  “chmod” – change file access permission chmod 744 calculate.sh “passwd” – update a user’s authentication tokens, “who” – show who is logged on, • “su” – change user ID or become super-user.  Basic Linux commands: • “scp” – secure copy (remote file copy program) • Scp copies files between hosts on a network • Usage: scp [OPTIONs] [[user]@host1:file1] [[user]@host2:file2]. Cross-compilation: arm-linux-gnueabihf-gcc main.c -o helloworld. Difference from Linux systems: Embedded Linux:  Purpose of using Linux kernel is to perform particular function. Desktop Linux: Purpose of using Linux kernel to many works for user. Basic C programming: Elements of a C program: Why C? Fast execution • Easy memory management programming • Bit operation • But a bit complex concepts of pointer, type conversion and memory allocation. What is gcc? gcc • stands for GNU C/C++ Compiler local compilation “gcc -Wall test.c -o test”. How to optimize a program by using gcc? -O, -O1, -O2, -O3, -O0, -Os • Various levels of optimization of the code • -O1 to -O3 are various degrees of optimization targeted for speed • If -O is added, then the code size is considered • -O0 means “no optimization” • -Os targets generated code size (forces not to use optimizations resulting in bigger code) Makefile-   Makefile describes to the make command that how to compile the program. Provide a way for separate compilation • Describe the dependencies among the project files  •  allows us to only compile those that have changed and the modules that depend upon them. “make –f filename“ gdb: Debugger 1. compile program • “gcc –g test.c –o test” • 2. start GDB • “gdb test” • 3. set breaking point • break main • 4. run program • “run” • 5. program execution continue to the next source line : “next” • 6. resume program execution : “continue”. GDB Commands • list(l)：list source code • break(b) function • break(b) line • break(b) file: line • break (b) line if condition • e.g., break 6 if i=10. • Info break : check all possible breakpoints • delete [breakpoint number]: delete # breakpoint • Without arguments deletes all breakpoints • run(r): run program • next(n): program execution continue to the next source line, omitting function calls • step(s): program execution continue to next source line, going into function calls • continue(c): resume program execution. print(p) variable ： check the value of a variable • finish： forced return (from a function) • watch variable： stops whenever the value of the variable changes • quit(q)：quit GDB. How to generate a uniform random variable?: 1.0*rand() / RAND_MAX (requires #include <stdlib.h>) How to use array? How to pass array to a function?: int find_largest(int a[], int n) { … } How to get user input from command line? int main (int argc, char *argv[]) { ... } Pointer: void swap(int *px, int* py) { int temp; temp = *px; *px = *py; *py = temp; } Structure: typedef struct { char *face; char *suit; } Card; Card deck[52]; Dynamic memory management: Malloc: line = (char*)malloc(line length); Takes number of bytes to allocate as argument. • Use sizeof to determine the size of a type. free(line); Calloc: void *calloc(size_t nitem, size_t size); requires two arguments - the number of variables you'd like to allocate memory for and the size of each variable. allocated memory space are zero by default. Realloc: If you find you did not allocate enough space use realloc(). If you find you did not allocate enough space use realloc(). ip = (int*)malloc(100 * sizeof(int)); /* need twice as much space */ ip = (int*)realloc(ip, 200 * sizeof(int));

BS: Which one is NOT one of the four phases in the compilation system? Compilation, Linking, Preprocessing, Assembly, Processing. Which command is used to measure the running time of a C program in Linux. time ./hello. The primary purpose of an operating system is to make the most efficient use of the computer hardware. What is phony target in the makefile? The target which is not a filename. Using the standard library function can be more efficient because “all of the above”. Which of the following must every C program have? Main. Lines beginning with a # are processed at preprocessor time. Sensors: Provide simple information , example photoresistor - /light intensity ADC: Analog to Digital converter, converts detected analog signal into a digital form for processing. Microcontroller: An integrated circuit that executes a program. Traditionally slower than a microprocessor (<500 MHz) (sends commands, receives data, need programming). IP Core: Circuit that performs one function. FPGA: Field Programmable gate array, Hardware controlled via RAM, faster than software. DAC: Converts Digital to analog. Acututor; causes events to occurs in the environment (LEDS, LCD, 7-seg display, servo motor).

Properties of TCP: Connection-oriented: setup required between client and server processes; Reliable transport between sending and receiving process; Flow control: sender won’t overwhelm receiver; Congestion control: throttle sender when network overloaded; Does not provide: timing, minimum throughput guarantees, security; Example: Web, SSH, SMTP (sending mail), IMAP/POP (receiving mail) Properties of UDP: Unreliable data transfer between sending and receiving process Does not provide: connection setup, reliability, flow control, congestion control, timing, throughput guarantee, or security; Example: Media steaming (lost frame are ok), Games that don’t care if you get every update Why is Networking Needed? To enhance many devices, To access data or computational power outside of the device

| <!DOCTYPE html> <html> <head> <title>Page Title</title> </head> <body> <a href="https://www.w3schools.com/html/">Visit our HTML tutorial</a> <!--comment --> <img src="pulpitrock.jpg" alt="Mountain View" width="500" height="377"> <h1>This is a Heading</h1> <p>This is a paragraph.</p> <table style="width:100%"> <tr> <th>First Name</th> </tr> <tr> <td>Jill</td> </tr> </table> <frameset rows = "10%,80%,10%"> <frame name = "top" src = "/html/top_frame.htm" /> <frame name = "main" src = "/html/main_frame.htm" /> <frame name = "bottom" src = "/html/bottom_frame.htm" /> <noframes> <body>Your browser does not support frames.</body> </noframes> </frameset> </body> </html> | /****************mult_v1.c**********/ #include <stdio.h> #include <stdlib.h> int main(void) { char *data; long m,n; printf("%s%c%c\n","Content-Type:text/html;charset=iso-8859-1",13,10); printf("<TITLE>Multiplication Operation</TITLE>\n");printf("<form><div><label>Multiplicand 1: <input name=\"m\" size=\"5\"></label></div>\n"); printf("<div><label>Multiplicand 2: <input name=\"n\" size=\"5\"></label></div>\n"); printf("<div><input type=\"submit\" value=\"Multiply!\"></div></form>"); data = getenv("QUERY_STRING"); if(data == NULL) printf("<P>"); else if(sscanf(data,"m=%ld&n=%ld",&m,&n)!=2) printf("<P>Error! Invalid data. Data must be numeric."); elseprintf("<P>The product of %ld and %ld is %ld.",m,n,m*n); return 0; //TCPClient import socket import sys sock = socket.socket(socket.AF_INET, socket.SOCK_STREAM) address = raw_input("ip address: ") port = input("port number: ") sock.connect((address, port)) try: while True: message = raw_input(">> "); sock.sendall(message) finally: sock.close() | b % a = 0 a**b =10 to the power 20 (!=) == >= <= less than ~ Binary Ones Complement for num in range(10,20): (10-19) for num in range(4,10,2): (4,6,8) print '%d equals %d * %d' % (num,i,j) len([1, 2, 3]) = 3 [1, 2, 3] + [4, 5, 6] = [1, 2, 3, 4, 5, 6] ['Hi!'] * 4 = ['Hi!', 'Hi!', 'Hi!', 'Hi!'] 3 in [1, 2, 3] = True for x in [1, 2, 3]: print x = 1 2 3 def printme( str ): print str return; //TCPServer import socket import sys sock = socket.socket(socket.AF_INET, socket.SOCK_STREAM) address = socket.gethostbyname(socket.gethostname()) port = input("port number: ") sock.bind((address,port)) sock.listen(1) while True: connection, client_address = sock.accept() try: while True: data = connection.recv(16) if data: print data Else: break finally: connection.close() |
| --- | --- | --- |

OSI Model: Application - Network Process to Application -> Presentation - Data representation and Encryption -> Session - Interhost communication -> Transport - End-to-End connection and Reliability -> Network - Path Determination and IP (Logical Addressing) -> Data Link - MAC and LLC (Physical addressing) -> Physical - Media, Signal and Binary Transmission.

CGI Flow - (Browser -> Server) ) 1. Browser requests Document, (Server -> CGI ) 2a. Server invokes CGI, (CGI Program -> Data) 2b. CGI executes and accesses disk or data center as needed, (CGI Program -> Server) 2c. CGI sends “html” to the server (Server -> Browser)  3. Server returns “html” formatted file to browser.

| //UDPServer import socket import sys sock = socket.socket(socket.AF_INET, socket.SOCK_DGRAM) port = input("host port: ") sock.bind(('',port)) while True: data, addr = sock.recvfrom(1024) if data == 'stop': break else: print "From addr: '%s', msg: '%s'" % (addr[0], data) sock.close() | //UDPClient import socket import sys sock = socket.socket(socket.AF_INET, socket.SOCK_DGRAM) address = raw_input("host ip address: ") port = input("host port: ") while True: data = raw_input('>> ') if len(data) == 0: break else: if sock.sendto(data, (address, port)): print "sending message" sock.close() |
| --- | --- |

Round Robin						FCFS